Tab 1 - Vehicle Management

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (Vehicle Control, Critical, Taxi sheets)

Packet bytes: VC = Vehicle Control byte = pkt[13];  MIX = Mixed Data byte = pkt[14];  MB2 = Mixed Byte 2 = pkt[23];  VM = Vehicle Mode = pkt[18].

## 1.  Tab 1 controls -> spec command

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Mode x9 (AUTO..SIGHTLINE SLV) | VehicleMode | VM (pkt[18]) | STANDBY=3 | Mutually-exclusive mode select | OK - values verified |
| Pilot Aug | PilotAugmentationOff | MIX . 4 | OFF | 1 = turn OFF pilot augmentation | OK (ON = aug disabled) |
| Full Throttle | fullThrottle | MIX . 3 | OFF | 1 = throttle to full | OK |
| Search | Search | MIX . 2 | OFF | 1 = initiate search pattern | OK |
| Alt: GPS | UseGPSAlt | VC . 5 | ON | 1 = use GPS altitude | OK |
| Alt: Pres Alt | UsePressAlt | VC . 6 | OFF | 1 = use pressure altitude | OK |
| Alt + | Ascend | VC . 1 | - | 1 = increase altitude cmd | OK (momentary) |
| Alt - | Descend | VC . 2 | - | 1 = decrease altitude cmd | OK (momentary) |
| Speed: GPS | Use_GPSSpeed | VC . 0 | GPS | 1 = GPS speed, 0 = ADS | OK |
| Speed: ADS | clears Use_GPSSpeed | VC . 0 = 0 | - | ADS selected when bit 0 = 0 | ISSUE - also sets UseAduSpeed (wrong bit) |
| Speed + | AscendSpeed | VC . 3 | - | increment climb speed | ISSUE - spec: send once |
| Speed - | DescndSpeed | VC . 4 | - | decrement climb speed | ISSUE - spec: send once |
| Brakes | Release_Brakes | MB2 . 7 | - | 1 = brakes released | OK |

## 2.  Key findings

-  Mode byte and values VERIFIED. The 9 mutually-exclusive mode buttons write VehicleMode (pkt[18]). Against the Mode Definition list, all values match: AUTO=0, SELFTEST=1, RUNUP=2, STANDBY=3 (default), MANUAL_EC=4, MANUAL_GCS=5, SEMIAUTO_HEADING=6, SEMIAUTO_ROLL=7, SIGHTLINE_SLAVE=8.

-  Most controls match and are wired: Pilot Aug (PilotAugmentationOff), Full Throttle (fullThrottle), Search, Alt GPS/Pres (UseGPSAlt/UsePressAlt), Alt +/- (Ascend/Descend), Speed GPS (Use_GPSSpeed), and Brakes (Release_Brakes).

-  Speed ADS sets the wrong extra bit. The spec models the speed sensor as a single bit (Use_GPSSpeed: 1 = GPS, 0 = ADS). ADS correctly clears that bit, but it ALSO sets UseAduSpeed, which maps to AP Byte 1 bit 5 - a bit the Auto Pilot sheet assigns to UseHeightErrLeadFilter. ADS should just clear Use_GPSSpeed.

-  Speed +/- should be send-once. AscendSpeed / DescndSpeed carry the remark "GCS must send this flag once for one time inc/dec." The +/- nudge buttons are momentary (repeat every packet while held), so a single press can step speed multiple times. They should be one-shot, like the Tab 5 waypoint Next/Previous. (Alt +/- have no such caveat - momentary is fine there.)

Tab 2 - Flight Automation (Auto Pilot Byte)

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (Auto Pilot sheet)

Packet bytes: AP1 = pkt[9];  AP2 = pkt[10];  AP3 = pkt[19]. "Byte.Bit" is the byte group and bit number (0-7).

## 1.  Tab 2 buttons -> spec command

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Ht Err Lead | UseHeightErrLeadFilter | AP1 . 5 | 0 | Use lead filter in height control loop | wired? NO (empty callback) |
| Init Pitch | InitPitchIDCmd | AP2 . 0 | 0 | Initiate ID waveform, pitch dynamics | wired? NO (empty callback) |
| Init Rudder | InitRudderIDCmd | AP2 . 2 | 0 | Initiate ID waveform, yaw dynamics | wired? NO (empty callback) |
| Init Aileron | InitAileronIDCmd | AP2 . 1 | 0 | Initiate ID waveform, roll dynamics | wired? NO (empty callback) |
| PR Ld Filter | UsePitchRateLeadFilter | AP2 . 3 | 0 | Use lead filter in pitch-rate loop | wired? NO (empty callback) |
| Def Lat Gains | SetDefaultLatGains | AP1 . 3 | 1 | Reset all lateral gains | OK (default ON matches spec) |
| Def Lon Gains | SetDefaultLonGains | AP1 . 2 | 0 | Reset all longitudinal gains | OK (verify default: spec=0, UI=ON) |
| Air Mode | AirMode | AP3 . 5 | 0 | 1 = Air modes enabled | OK |
| SLT | ExecuteSLT | AP3 . 1 | 0 | 1 = Start SLT | wired? NO (empty callback) |
| Flaps Down | FlapsDown | AP3 . 0 | 0 | 1 = Flaps down | OK |
| NonLin Roll | Use_NonLinear_RollControl | AP2 . 6 | 0 | 1 = Non-linear roll control | OK |
| Flaps/Ailrn | Use_Flaps_As_Ailerons | AP2 . 7 | 0 | 1 = Flaps used as ailerons | OK |
| Alt Hold | AltitudeHold | AP3 . 2 | 0 | 1 = Altitude hold ON | OK |
| Gnd Clear | GroundClearance | AP3 . 6 | 0 | 1 = Ground clearance given | OK |
| Tel Logging | EnableTelLogging | AP3 . 4 | 0 | 1 = Telemetry logging enabled | OK |
| Speed Hold | SpeedHold | AP3 . 3 | 0 | 1 = Speed hold ON | OK |
| Pitch Angle | PitchControlScheme = 0 | AP1 . 4 | 0 | 0 = NORMAL scheme (pitch angle) | OK (via HeightControlScheme) |
| ROC Control | PitchControlScheme = 1 | AP1 . 4 | 0 | 1 = ROC control | OK (via HeightControlScheme) |
| Gain Tune (chk) | EnableGainTuning | AP2 . 4 | 0 | 1 = Gain tuning from GCS enabled | OK |
| Stability Augmentation (chk) | EnableStabilityAugmentation | AP2 . 5 | 0 | 1 = Stability augmentation enabled | OK |

## 2.  Key findings

-  Two phantom buttons removed. "HT Err Intg" and "Thr Err Intg" had no match in the workbook and have been removed; the grid was reflowed.

-  Several buttons are wired to empty callbacks. Ht Err Lead, Init Pitch, Init Rudder, Init Aileron, PR Ld Filter and SLT map to real spec bits but currently send nothing.

-  "Def Lon Gains" default. Spec default is 0 (OFF); UI defaults it ON. Only SetDefaultLatGains has a spec default of 1.

## 3.  Spec AP commands not on Tab 2

Per the Auto Pilot sheet these commands live in the AP byte, but are on other tabs (or not built yet):

| Command Variable | Pkt Byte.Bit | Dft | Meaning | Where it is now |
| --- | --- | --- | --- | --- |
| SelectIncDecParam | Byte 17 | 0 | Selects parameter 1-255 to inc/dec (grid + text box) | Not built |
| IncParam | AP1 . 0 | 0 | 1 = increment selected parameter | Not built |
| DecParam | AP1 . 1 | 0 | 1 = decrement selected parameter | Not built |
| SwitchTel Bit 0 | AP1 . 6 | 0 | FCC selection (Default/Master/Slave) | On Tab 4 (FCC Selection) |
| SwitchTel Bit 1 | AP1 . 7 | 0 | FCC selection (Default/Master/Slave) | On Tab 4 (FCC Selection) |
| EnableSysID | AP3 . 7 | 0 | 1 = System ID enabled | On Tab 4 (System ID checkbox) |

Tab 3 - Landing Control (Landing Byte)

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (Landing sheet)

Packet bytes: LB = Landing Byte = pkt[11];  VC = Vehicle Control byte = pkt[13].

## 1.  Tab 3 controls -> spec command

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Takeoff Direction (radio) | TODirection | LB . 0 | 0 | Select takeoff direction | OK |
| Landing | Landing | LB . 1 | 0 | 1 = Initiate landing approach | OK |
| Landing Direction (radio) | LandingDirection | LB . 2 | 0 | Select landing direction | OK |
| Dummy Landing | DummyLanding | LB . 3 | 0 | 1 = Dummy landing (ATL test) | OK |
| Abort Landing | AbortLanding | LB . 4 | 0 | 1 = Abort landing | OK |
| Deny Hit Crit | DenyHitCriteria | LB . 5 | 0 | Ignore landing window hit criteria | OK |
| Emrg Landing | EmergencyLandingFlag (EmergencyLanding) | LB . 6 | 0 | 1 = Emergency landing sequence | OK |
| Retract / Deploy | RetractNoseLandingGearCMD (NLRetractionCmd) | LB . 7 | 0 | 1 = Retract NLG, 0 = Deploy NLG | FIXED (now NLRetractionCmd) |

## 2.  Key findings

-  All 8 Landing-byte commands are present and correctly represented (TODirection, Landing, LandingDirection, DummyLanding, AbortLanding, DenyHitCriteria, EmergencyLandingFlag, RetractNoseLandingGearCMD).

-  Retract / Deploy wiring FIXED. Previously set RetractNoseLandingGear (Auxiliary byte, pkt[15] bit 1). Now sets NLRetractionCmd (Landing byte bit 7) to match the spec's RetractNoseLandingGearCMD. Retract=1, Deploy=0, default Deploy.

-  "Radar Alt" is intentionally on Landing. The radar altimeter is enabled during landing as the height reference, so its placement here is by design. Note only that the bit it sets (UseRadarAlt) lives in the Vehicle Control byte, not the Landing byte.

Tab 4 - System ID & FCC

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (Auxiliary, Mixed-Byte-2, SYS ID Byte, Auto Pilot sheets)

Packet bytes: AUX = Auxiliary byte = pkt[15];  MB2 = Mixed Byte 2 = pkt[23];  SID = SYS ID Byte = byte 22 (pkt[20], NYI);  AP3 = pkt[19].

## 1.  Tab 4 controls -> spec command

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Payload Video | SelectPayloadVideo | AUX . 4 | 0 | 1 = payload video, 0 = nose camera | Unwired (needs field) |
| Deny Abort Taxi | Deny_Abort_Taxi | MB2 . 0 | 0 | 1 = GCS override to deny abort taxi | OK |
| Radio Silence | RadioSilence | AUX . 5 | 0 | 1 = turn off all communications | Unwired (needs field) |
| Backup Link | SwitchToBckUpLink | AUX . 0 | 0 | 1 = switch to backup link | OK |
| FCC: Master | Use FCC = 0 | AUX . 6 | 0 | 0 = Master | FIXED (now UseFcc) |
| FCC: Slave | Use FCC = 1 | AUX . 6 | 0 | 1 = Slave | FIXED (now UseFcc) |
| SysID matrix (Reset/DC, Chirp, Doublet) | SYS ID Byte - low nibble (sequence) | SID (byte 22) | 0 | Sequence: Sin/Chirp/Doublet/etc. | Visual only (byte 22 NYI) |
| Elevator/Aileron/Rudder/Throttle/Reset | SYS ID Byte - high nibble (command) | SID (byte 22) | 0 | Command: Elev/Ail/Rud/Throttle | Visual only (byte 22 NYI) |
| System ID (checkbox) | EnableSysID | AP3 . 7 | 0 | 1 = System ID enabled | Unwired (needs field) |
| DGPS Corrections | DGPS_Corr_Enabled | MB2 . 1 | 0 | 1 = DGPS corrections enabled | OK |
| OVI Data | (external interface) | - | - | Manual pilot input: joystick, rudder pedals, throttle dock | External - not a command bit (later) |
| Payload | (external interface) | - | - | UDP + serial-port interface | External - part of later integration |

## 2.  Key findings

-  FCC Selection FIXED. Per the Auxiliary sheet, "Use FCC" (byte 15 bit 6) = 0 Master, 1 Slave. The old Default/Master/Slave group (which wrongly used EnableFccOverride = "turn off power to payload") is replaced by a Master/Slave toggle on UseFcc, default Master.

-  SwitchTel is telemetry, not FCC. The Auto Pilot Default/Master/Slave truth table applies to SwitchTel (Switch Telemetry, AP byte bits 6-7), a separate command from FCC Master/Slave.

-  Real commands still unwired. Payload Video (SelectPayloadVideo, AUX.4), Radio Silence (RadioSilence, AUX.5) and System ID (EnableSysID, AP3.7) exist in the spec but have no CommandState field yet.

-  System ID Byte grouping is correct. Byte 22 packs a sequence in the low nibble (Reset/DC, Chirp, Doublet) and a surface command in the high nibble (Elevator/Aileron/Rudder/Throttle). Encoding is not implemented (byte 22 = 0).

-  "Deny Sensors Fail" removed. No matching command existed in the workbook; the button has been removed (its Row 0 / Col 0 slot left empty).

-  OVI Data and Payload are external interfaces, not command bits. OVI Data is live manual pilot input (joystick, rudder pedals, throttle dock). Payload is a UDP + serial-port interface. Neither maps to a GCS command byte; both will be wired during later integration and are left as unwired placeholders for now.

Tab 5 - Navigation

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (Navigation sheet)

Packet bytes: NAV = Navigation byte = pkt[12];  MIX = Mixed Data byte = pkt[14].

## 1.  Tab 5 controls -> spec command

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Next (Way Point) | GoToNextWayPoint | NAV . 0 | 0 | Increment destination waypoint | OK - momentary (send-once) |
| Previous (Way Point) | GoToPrevWayPoint | NAV . 1 | 0 | Decrement destination waypoint | OK - momentary (send-once) |
| Left (Track Corr.) | LeftXTrackCorrection | NAV . 2 | 0 | Subtract offtrack correction | OK |
| Right (Track Corr.) | RightXTrackCorrection | NAV . 3 | 0 | Add offtrack correction | OK |
| Use Curve Guidance | UseCurveGuidance | NAV . 4 | 0 | 1 = use curve guidance scheme | OK |
| Offtrk Linear | LateralGuidanceScheme = 0 | NAV . 5-6 | 0 | 0 = Offtrack Linear scheme | OK |
| Offtrk NonLinear | LateralGuidanceScheme = 1 | NAV . 5-6 | 0 | 1 = Offtrack Nonlinear scheme | OK |
| Pursuit Guid | LateralGuidanceScheme = 2 | NAV . 5-6 | 0 | 2 = Pursuit guidance | OK |
| Loiter | Loiter | NAV . 7 | 0 | 1 = Start loitering at current position | OK |
| R2 Base | R2Base (GcsR2Base) | MIX . 1 | 0 | 1 = Return to base | OK |
| Dash R2 Base | DashR2Base | MIX . 0 | 0 | 1 = Full-throttle return to base | OK - newly wired |

## 2.  Key findings

-  All 11 controls match real spec commands - no phantom buttons and no mis-wires. Tab 5 is fully aligned with the Navigation sheet.

-  Next / Previous made momentary (send-once). The sheet warns waypoints increment for every packet the flag is held. Each click now sets the bit, transmits ONE packet via SendNow(), then clears it - so it steps exactly one waypoint per click and can never latch.

-  Dash R2 Base now wired. Added CommandState.DashR2Base and encoded it into the Mixed Data byte bit 0 (pkt[14] 0x01). R2 Base stays on bit 1 (GcsR2Base).

Tab 6 - Power / Servo (Relay Control Bytes)

Command-variable mapping vs. "GCS Cmd Packet Structure.xlsx" (RelayCntrlByte1/2/3 sheets)

Packet bytes: RB1 = RelayControlByte1 = pkt[26];  RB2 = pkt[27];  RB3 = pkt[28].  Polarity per spec: 0 = ON/Enable, 1 = OFF/Disable.

## 1.  Tab 6 buttons -> spec relay bit

| Button (UI) | Command Variable | Pkt Byte.Bit | Dft | Meaning | Status / Notes |
| --- | --- | --- | --- | --- | --- |
| Aileron (Port) | Port_Aileron | RB1 . 7 | ON | Servo | OK |
| Flap (Port) | Port_Flap | RB1 . 1 | ON | Servo | OK |
| B Rudder (Port) | Port_Bottom_Ruddervator | RB1 . 6 | ON | Servo | OK |
| Top Rudder (Port) | Port_Top_Ruddervator | RB1 . 0 | ON | Servo | OK |
| Video Rec | Video_Recorder | RB1 . 3 | OFF | Servo (spec dft ON) | OK - UI default |
| Strobe | Strobe_Light | RB1 . 2 | OFF | Servo (spec dft ON) | OK - UI default |
| Aileron (Stbd) | StarBoard_Aileron | RB1 . 4 | ON | Servo | OK |
| Flap (Stbd) | StarBoard_Flap | RB2 . 6 | ON | Servo | OK |
| B Rudder (Stbd) | StarBoard_Bottom_Ruddervator | RB1 . 5 | ON | Servo | OK |
| Top Rudder (Stbd) | StarBoard_Top_Ruddervator | RB2 . 7 | ON | Servo | OK |
| N Wheel | Nose_Wheel | RB2 . 5 | ON | Servo | OK |
| Servo12 | Servo12_Cntrl | RB2 . 4 | ON | Servo | OK |
| Weapon | Weapon_Cntrl | RB2 . 3 | OFF | 0=ON, 1=OFF | OK |
| RETRC | Retraction | RB3 . 3 | OFF | 0=Enable, 1=Disable | OK |
| ComLink | ComLnk | RB3 . 7 | ON | 0=ON, 1=OFF | OK |
| Camera OPT12 | Camera/Opt12 | RB3 . 6 | ON | 0=ON, 1=OFF | OK |
| Pitot Heater | Pitot Heater_Cntrl | RB2 . 1 | OFF | 0=ON, 1=OFF | OK |
| Payload | Payload_Cntrl | RB2 . 2 | OFF | 0=ON, 1=OFF | OK |
| AUX 1 | Aux1 | RB3 . 0 | OFF | Relay | OK |
| OPT5V | Opt5v | RB3 . 1 | ON | 0=ON, 1=OFF | OK |
| Brake Power | Brakes | RB3 . 2 | OFF | 0=Enable, 1=Disable | OK |
| ALTERNATOR | Alternator | RB3 . 4 | ON | 0=ON, 1=OFF | OK |
| AUX 2 | Aux2 | RB3 . 5 | OFF | 0=ON, 1=OFF | OK |
| ECU | ECU_Cntrl | RB2 . 0 | ON | 0=ON, 1=OFF | OK |

## 2.  Key findings (after re-map)

-  All 24 bit positions now match the spec (verified against the three RelayCntrlByte sheets). The earlier sequential guess was replaced with the exact byte/bit for every relay, including the correct byte grouping (e.g. Stbd Bottom Rudder and Stbd Aileron are in Byte 1; ECU in Byte 2; ComLnk/Camera/Aux/Alternator/Retraction/Brakes in Byte 3).

-  Polarity corrected in SetRelay. The spec is inverted (0 = ON/Enable, 1 = OFF/Disable). A button turned ON now clears its bit; OFF sets it.

-  Defaults: 22 of 24 match the spec. The only deviations are Video Rec and Strobe, kept OFF as an intentional UI choice (spec default ON).

-  Seed bytes: RB1 = 0x0C (Video Rec + Strobe OFF), RB2 = 0x0E, RB3 = 0x2D - all consistent with the button states.

-  Telemetry readouts unchanged. Brakes Actuation and NLG Retraction are telemetry values (text), not command bits.