# GCS_240626 — Tab 1 (Vehicle Management) Command Packet Mapping

Document describes the relationship between Tab 1 UI buttons and GCS command packet variables.

Packet structure reference: GCS Cmd Packet Structure.xlsx

## 1. Vehicle Mode Buttons — 3 × 3 Grid (Rows 0–2)

Nine mutually exclusive buttons write to the VehicleMode byte (Byte 18, pkt[18], CommandState.VehicleMode).

Only one mode may be active at a time. STANDBY (value 3) is the startup and RESET default.

| Button Label | Mode Value | CommandState Field | Packet Location | Notes |
| --- | --- | --- | --- | --- |
| AUTO | 0 | VehicleMode | Byte 18 (pkt[18]) | STANDBY is default |
| SELF TEST | 1 | VehicleMode | Byte 18 (pkt[18]) |  |
| RUNUP | 2 | VehicleMode | Byte 18 (pkt[18]) |  |
| STANDBY | 3 | VehicleMode | Byte 18 (pkt[18]) | Default ON |
| MANUAL EC | 4 | VehicleMode | Byte 18 (pkt[18]) |  |
| MANUAL GCS | 5 | VehicleMode | Byte 18 (pkt[18]) |  |
| SEMIAUTO HEADING | 6 | VehicleMode | Byte 18 (pkt[18]) |  |
| SEMIAUTO ROLL | 7 | VehicleMode | Byte 18 (pkt[18]) |  |
| SIGHTLINE SLAVE | 8 | VehicleMode | Byte 18 (pkt[18]) |  |

## 2. Row 3 Buttons

Three independent toggle buttons. 'Default' column shows initOn condition as specified.

| Button Label | Default | CommandState Field | Packet Location | Notes |
| --- | --- | --- | --- | --- |
| Pilot Augmentation ON | true | PilotAugmentationOff | Byte 14 (pkt[14]) Bit 4 — 0x10 | Inverted: button ON → bit=0 (aug enabled). Button OFF → bit=1 (aug disabled). |
| Full Throttle ON | true | FullThrottle | Byte 14 (pkt[14]) Bit 3 — 0x08 | Button ON → full throttle engaged. |
| Search OFF | false | Search | Byte 14 (pkt[14]) Bit 2 — 0x04 | Default OFF. Button ON → initiate search pattern. |

## 3. Sensor Selector GroupBoxes (Do Not Change)

Mutually exclusive within each group. GPS is the default for both altitude and speed.

| Button / Option | Default | CommandState Field | Packet Location | Notes |
| --- | --- | --- | --- | --- |
| Altitude — GPS (default) | true | UseGpsAlt | Byte 13 (pkt[13]) Bit 5 — 0x20 | Mutually exclusive with Pres Alt. |
| Altitude — Pres Alt | false | UsePresAlt | Byte 13 (pkt[13]) Bit 6 — 0x40 | Mutually exclusive with GPS. |
| Speed — GPS (default) | true | UseGpsSpeed | Byte 13 (pkt[13]) Bit 0 — 0x01 | Mutually exclusive with ADS. |
| Speed — ADS | false | (UseGpsSpeed=0) | Byte 13 (pkt[13]) Bit 0 — 0x01 | Complement of UseGpsSpeed. |

## 4. Brakes Button

Single toggle below sensor groupboxes. Default ON (brakes applied). Inverted logic relative to ReleaseBrakes bit.

| Button Label | Default | CommandState Field | Packet Location | Notes |
| --- | --- | --- | --- | --- |
| Brakes | true (applied) | ReleaseBrakes | Byte 23 (pkt[23]) Bit 7 — 0x80 | Inverted: button ON → brakes applied → ReleaseBrakes=false (not released). |

## 5. Packet Byte Reference Summary

| pkt[ ] | Byte Description | Key Fields in Tab 1 | Notes |
| --- | --- | --- | --- |
| pkt[13] | Vehicle Control Byte | UseGpsSpeed, UseGpsAlt, UsePresAlt | Byte 13 in packet / Byte 9 in CommandState comments |
| pkt[14] | Mixed Data Byte | Search (0x04), FullThrottle (0x08), PilotAugmentationOff (0x10) | Byte 14 in packet / Byte 10 in CommandState comments |
| pkt[18] | VehicleMode | VehicleMode (0–8) | Direct byte value — no bit masking |
| pkt[23] | Mixed Byte 2 | ReleaseBrakes (0x80) | Also holds DenyAbortTaxi, EnableTelemetryLink etc. |

Generated: 17 Jul 2026   |   GCS_240626 project

## 6. Alt & Speed Nudge Buttons (+/−)

Four small momentary-action buttons (50 × 25 px each — half the width and height of the GPS/Pres Alt buttons) placed to the right of the Altitude and Speed sensor groupboxes. Holding a button down asserts the corresponding bit; releasing clears it. These are implemented via MouseDown/MouseUp, not a toggle Click, because the FCC treats them as transient commands.

| Button | Groupbox | Behaviour | CommandState Field | Packet Location | Notes |
| --- | --- | --- | --- | --- | --- |
| + | Altitude Sensor In Use | Hold = ascend | GcsAscend | pkt[13] Bit 1 — 0x02 | Top button, right of Alt groupbox |
| − | Altitude Sensor In Use | Hold = descend | GcsDescend | pkt[13] Bit 2 — 0x04 | Bottom button, right of Alt groupbox |
| + | Speed Sensor In Use | Hold = increment speed | GcsIncrementSpeed | pkt[13] Bit 3 — 0x08 | Top button, right of Speed groupbox |
| − | Speed Sensor In Use | Hold = decrement speed | GcsDecrementSpeed | pkt[13] Bit 4 — 0x10 | Bottom button, right of Speed groupbox |

All four bits are in pkt[13] Vehicle Control Byte (TempBuff[9]). Button size is 50 × 25 px. Wire: MouseDown → State.Field = true; MouseUp → State.Field = false.